This process is for continuously eliminating bacteria inside your

dental waterlines and is therefore called a ‘prevention’ cleaning

process.

It is one of 2 methods recommended for complete waterline

disinfection.

On first time of use, carry out the biofilm ‘removal’ shock process

first.

Dilute PEROXY AG+ at 2%. Eg 20ml of PEROXY AG+ in 1L bottle and

980ml bacteria-free purified water. Mix well.

Replace your water bottle, pull through the diluted PEROXY AG+ and

use continuously throughout the day in your waterlines.

At the end of the day or week:

• remove handpieces & couplings & send to autoclave

• empty your water bottle of any remaining diluted

PEROXY AG+, refill with purified or tap water and flush

through all waterlines for 10 seconds

• then purge waterlines of any liquid and allow to dry

overnight

• now remove your water bottle and allow it to dry inverted

(HTM01/05 sec 6.83 & 19.11)

• pour away unused diluted PEROXY AG+ down the sink

1

2

3

4

5

6

Repeat the process daily when the

practice is open. 7

It is important to carry out the

‘removal’ (shock) procedure after

the following:

• Periodically when used with this

continuous dosing treatment

(typically monthly, then quarterly)

• Whenever a bacterial test (TVC) fails

on the waterlines

• After an extended period of inactivity

in the practice (eg renovations,

lockdown, holidays etc)

8

IT IS IMPORTANT TO REMEMBER:

• To flush your 3-in-1 waterline

BETWEEN patients for 20-30

seconds (HTM01/05 sec 6.85)

• To monitor bacteria in the

waterlines every 3 months, using

either a specialist water lab or a

TVC dipslide.

9